ECE3051 – Analog and Digital Signal Processing, Fall Semester 2022-2023

ELA DA – 5, Slot: L25-L26

By: Jonathan Rufus Samuel (20BCT0332) Date: 13.11.2022

__________________________________________________________________________________

ELA DA 5 – DOS: 13.11.2022

Task - 5: Realization of Adaptive Differential Pulse Code Modulation (ADPCM)

Q1) Using Abdominal and Direct Foetal ECG Database from Physionet, perform the following subtasks:

- Sub-Task 1 - Plot the ECG Graph

- Sub-Task 2 - Analyse the stationarity of the given ECG signal through appropriate waveforms

- Sub-Task 3 - Detect the diverse ECG abnormalities if any with respect to the given ECG signal

CODE:

%Task - 4: ECG Signal Analysis

%Name: Jonathan Rufus Samuel (20BCT0332)

%Course: ECE3051 - ELA

%DOS: 23.10.2022

%Using Abdominal and Direct Fetal ECG Database from Physionet, perform the

%following subtasks:

%Sub-Task 1 - Plot the ECG Graph

data_r1 = edfread("r01.edf","SelectedSignals","Direct_1");

data2_r1 = table2array(data_r1);

data3_r1 = vertcat(data2_r1{:});

subplot(2,1,1);

plot(data3_r1(1:5000));

title('Base ECG Signal - Direct1');

xlabel('time (t)');

ylabel('Magnitude (x(n))');

fetus_data_r1 = edfread("r01.edf","SelectedSignals","Abdomen_1");

fetus_data_r2 = table2array(fetus_data_r1);

fetus_data_r3 = vertcat(fetus_data_r2{:});

subplot(2,1,2);

plot(fetus_data_r3(1:5000));

title('Additional ECG Signal (from Fetus) - Abdomen1');

xlabel('time (t)');

ylabel('Magnitude (x(n))');

%Sub-Task 2 - Analyse the stationarity of the given ECG signal through

% appropriate waveforms:

plot(data3_r1(1:5000));

title('Base ECG Signal - Direct1');

xlabel('time (t)');

ylabel('Magnitude (x(n))');

%Stationarity: A stationary time series is one whose properties do not

% depend on the time at which the series is observed. As seen above,

% waveform changes w.r.t time, over intervals 1 - 5000. Therefore,

% Stationarity property does not hold for goven signal.

%Sub-Task 3 - Detect the diverse ECG abnormalities if any with respect to

% the given ECG signal:

%Samplling of Base Signal:

samp1 = data3_r1(1:300);

plot(samp1);

title('Sample ECG Signal - Direct1');

xlabel('time (t)');

ylabel('Magnitude (x(n))');

%Auto-Correlation between Sample & Base Signal:

plot(autocorr(data3_r1));

title('Auto-Correlation of Sample with Base Signal');

xlabel('time (t)');

ylabel('Magnitude (x(n))');

%Cross-Correlation between Base Signal & Fetus Signal:

plot(xcorr(data3_r1,fetus_data_r3));

title('Cross-Correlation of Sample with Base Signal');

xlabel('time (t)');

ylabel('Magnitude (x(n))');

OUTPUT:

>> %Task - 4: ECG Signal Analysis

%Name: Jonathan Rufus Samuel (20BCT0332)

%Course: ECE3051 - ELA

%DOS: 23.10.2022

%Using Abdominal and Direct Fetal ECG Database from Physionet, perform the

%following subtasks:

%Sub-Task 1 - Plot the ECG Graph

data_r1 = edfread("r01.edf","SelectedSignals","Direct_1");

data2_r1 = table2array(data_r1);

data3_r1 = vertcat(data2_r1{:});

subplot(2,1,1);

plot(data3_r1(1:5000));

title('Base ECG Signal - Direct1');

xlabel('time (t)');

ylabel('Magnitude (x(n))');

fetus_data_r1 = edfread("r01.edf","SelectedSignals","Abdomen_1");

fetus_data_r2 = table2array(fetus_data_r1);

fetus_data_r3 = vertcat(fetus_data_r2{:});

subplot(2,1,2);

plot(fetus_data_r3(1:5000));

title('Additional ECG Signal (from Fetus) - Abdomen1');

xlabel('time (t)');

ylabel('Magnitude (x(n))');

%Sub-Task 2 - Analyse the stationarity of the given ECG signal through

% appropriate waveforms:

plot(data3_r1(1:5000));

title('Base ECG Signal - Direct1');

xlabel('time (t)');

ylabel('Magnitude (x(n))');

%Stationarity: A stationary time series is one whose properties do not

% depend on the time at which the series is observed. As seen above,

% waveform changes w.r.t time, over intervals 1 - 5000. Therefore,

% Stationarity property does not hold for goven signal.

%Sub-Task 3 - Detect the diverse ECG abnormalities if any with respect to

% the given ECG signal:

%Samplling of Base Signal:

samp1 = data3_r1(1:300);

plot(samp1);

title('Sample ECG Signal - Direct1');

xlabel('time (t)');

ylabel('Magnitude (x(n))');

%Auto-Correlation between Sample & Base Signal:

plot(autocorr(data3_r1));

title('Auto-Correlation of Sample with Base Signal');

xlabel('time (t)');

ylabel('Magnitude (x(n))');

%Cross-Correlation between Base Signal & Fetus Signal:

plot(xcorr(data3_r1,fetus_data_r3));

title('Cross-Correlation of Sample with Base Signal');

xlabel('time (t)');

ylabel('Magnitude (x(n))');

>>

OUTPUT (SS):

__________________________________________________________________________________